CRABTREE INSULATION LIMITED

Method Statement/Risk Assessment

For the insulation of ductwork and heating,

domestic and chilled water pipework

at the site of

WAITROSE STORE

NEWPORT

FOR

ICH PROJECTS LTD.

Crabtree Insulation Limited

Unit 50 Globe Industrial Estate

Towers Road

Grays

Essex

RM17 6ST

Prepared by Sean Crabtree

Tel: 01375 370987

SCOPE OF THE WORKS

______________________________________________________________

Sales floor ductwork

Insulate ductwork with 40mm thick foil faced flexible mineral wool duct insulation secured with 100mm wide foil tape and aluminium bands. Fit IDs.

External ductwork

Insulate external ductwork with 40mm thick foil faced phenolic slab secured with 100mm wide foil tape. Cover with Proclad.

Internal heating, domestic water and chilled water pipework along top of cabinets

Insulate pipework with 25mm thick foil faced mineral wool insulation secured with 50mm wide foil tape. Fit ID bands.

External pipework

Insulate pipework with foil faced mineral wool insulation secured with foil tape. Cover with Proclad.

DAC Pipework

Paint pipework with two coats of paint.

Trace heating

Supply and fit trace heating to external cold water service pipework. Insulate with foil faced insulation and cover with black Proclad. Final electrical connections to be made by a qualified electrician.

SEQUENCE OF THE WORKS

_____________________________________________________________

Preparation of the work area:

The works required on site will be discussed with the management of the site to ensure that they are aware that works are to commence in areas, as access and egress may to some extent be restricted. Assurance will be gained to certify that the works area is prepared and will remain under his control.

All employees of Crabtree Insulation Limited will be inducted to the site on arrival by the Principal Contractor and will not be permitted to commence work until they are made aware of site hazards, location of welfare facilities, rules and the means of raising the alarm in the event of an emergency etc. Crabtree Insulation Limited will give their own induction, which will include details of this method statement and risk assessment and the controls required.

The works will be undertaken both outside of normal working hours and during normal working hours. There is a possibility that other trades may also be undertaking operations. The Principal Contractor will ensure that co-ordination of such operations should this occur.

Receiving/Taking delivery of materials & transporting to the works area:

Arrangements will be made with the Principal Contractor to ensure that an adequate set down area is supplied as close to the works area as is practicable in accordance with the Manual Handling Operations at Work Regulations 1992 and to ensure that there is no disruption to local or on-site traffic. Materials used by Crabtree Insulation Limited are of a light nature and will be manually carried to the work site in adequately reduced workloads and where necessary by two operatives in the prevention of manual handing associated incidents.

Materials will be carried to the work area and will not be stacked or stored in any area that restricts emergency access and egress routes. There is a negligible risk of injury to third parties due to the light nature of the materials but a minimum amount of materials will be carried at any one time. As outlined above all materials will be taken directly to the works area and will not be stored where they may reduce access and egress or result in a trip hazard. During the unloading of the vehicle all work equipment will be visually checked to ensure its integrity in accordance with the Provision and Use of Work Equipment Regulations 1998 and additionally operatives will make a second visual check before the use of equipment to ensure it is fit for purpose.

Undertaking the works:

The majority of insulation operations are to be undertaken 2-3 metres above floor level. Access in the main for these works will be via MEWPS. Steps may be utilised on occasion. Some works will be undertaken at floor level.

Electrical tools will not be required by Crabtree Insulation Limited.

Crabtree Insulation Limited will work closely with the Principal Contractor and will communicate their expected working areas at the start of the shift to ensure that, if required, due to reduced access and egress in the work areas, the Principal Contractor needs to plan alternate escape routes and to ensure that there is no overlap of works with third parties.

The Principal Contractor is to ensure that engineers and others are prevented from working in the work area of Crabtree Insulation Limited until the have completed their works.

The activities of Crabtree Insulation Limited are of a low risk but this risk will increase if the undertakings of others are allowed to be undertaken in the works area of Crabtree Insulation Limited.

Crabtree Insulation Limited will be undertaken to remove all waste from their works area as soon as there is a significant quality or a Health and Safety hazard is presented e.g. trips and slips or blocking of access/egress.

COMPLETION

Once the work is completed all tools and equipment will be cleared from the work area with materials placed into bins and skips provided by the Principal contractor by Crabtree Insulation limited.

P.P.E:

In accordance with the Personal Protective Equipment at Work (Amendment) Regulations 2022, Crabtree Insulation Limited will reduce the risks to employees as far as is reasonably practicable by the implementation of control measures within systems of work.

Should control measures not be available or they are deemed impractical, as a last resort, operatives will be provided with the necessary personal protective equipment.

The following PPE will be worn if it is considered necessary to control the hazards highlighted by this Method Statement and the Risk Assessments at the back of this document.

| Safety Footwear | High visibility clothing | Safety googles | Gloves | Ear protection | Respiratory Protection | Head Protection |
| --- | --- | --- | --- | --- | --- | --- |
| EN ISO 20345:2011 | EN ISO 20471 | BS EN 166:2002 | BS EN 388 A1:2018 | BS EN 352:2020 | BS EN 149:2001 | BS EN 397: 2012 |

Operatives will be supplied with PPE free of charge and will be informed of the necessity for the wearing and maintenance of the equipment. Suitable and sufficient clothing must be worn at all times e.g. no bare arms or legs or material soiled clothing.

Supervision:

Throughout the works Crabtree Insulation Limited will ensure that a supervisor is available permanently to oversee the works being undertaken on site. Works will be supervised to ensure that risks identified by method statement are controlled or are lowered so far as reasonably practicable and that further controls are implemented as required. Supervision by Crabtree Insulation Limited and the Principal Contractor will also ensue that all stipulations are upheld and complied with.

Welfare Facilities

Toilets, hot water, wash facilities and potable water are available within the confines of the site to the standard required by The Construction (Design and Management) Regulations 2015

and are available for use by operatives of Crabtree Insulation Limited.

Training

Operatives will be involved in a pre-works induction given by representatives of the Principal Contractor to outline site hazards, location of welfare facilities, location of first aid facilities, rules and the means of raising the alarm in the event of an emergency etc. Crabtree Insulation Limited will give their own induction, which will include this method statement to ensure that operatives are familiar with the method of works specific to this particular document and site conditions.

All personnel working for Crabtree Insulation Limited are familiar with the system to be used, the use of tools and equipment, and have undertaken a history of similar in all respects to the task being undertaken. The supervisor of Crabtree Insulation Limited will however highlight to operatives on site additional hazards that may be present in their immediate environment and will ensure when giving tasks and through supervision that operatives are competent to undertake them and that there are no additional hazards present

Any works undertaken within properties constructed prior to the year 2000 may contain Asbestos Containing Materials (ACMs). All members of our workforce have received Asbestos Awareness training and are competent in identifying materials that may contain asbestos. Should any suspected ACMs be identified, works will cease immediately, the area will be made safe, and the client/site management will be informed. No works will be carried out on or near suspected ACMs unless appropriate surveys, controls, and permissions are in place.

Safety of third parties

The protection of third parties will be considered before the commencement of all tasks on site. Access and egress will be kept clear at all times and the work areas have suitable signage and barriers erected to ensure the protection of others in accordance with the Health and Safety at Works act. Act 1974. where required. All loading or unloading of vehicles will be undertaken on site therefore the works of Crabtree Insulation Limited are not expected to impinge on the public in any way.

Where work is undertaken over small voids a check will be made to ensure that others are not present below or controls are reduce the risk of materials or tools etc. Dropping from height

Those persons on whom the undertakings of Crabtree Insulation Limited may impinge will be  provided with adequate warning of dangers that may be present and informed of actions that should be taken to minimise the risk. The system of work and how it may impinge on others will be considered at all times and controls such as barriers and signage will be used when appropriate

First Aid Facilities

First Aid requirement including the identity of the First Aider / Appointed person will be communicated to all operatives during induction. A First-Aider and box will be provided on site by the Principal Contractor adequately stocked for hazards highlighted in this method statement and for hazards that exist on a construction project, eyewash facilities will also be available on site, As a minimum requirement a telephone will be available within the site Managers offices with emergency number displayed and given to operatives at induction.

Emergency Procedures

Emergency procedures will be arranged on site will be communicated to all operatives during induction by the Principal Contractors representative, including the identity of the fire marshal and the location of the muster point and fire extinguishers.

Legislation

Crabtree Insulation Limited will undertake all works in compliance with current legislation

The Health and Safety at Work etc. Act 1974

The Management of Health and Safety at Work Regulations 1999 (as amended)

The Construction (Design and Management) Regulations 2015

The Control of Substances Hazardous to Health Regulations 2002 (as amended)

The Personal Protective Equipment at Work (Amendment) Regulations 2022

The Manual Handling Operations Regulations 1992

The Provision and Use of Work Equipment Regulations 1998

The Work at Height Regulations 2005 (as amended)

The Control of Noise at Work Regulations 2005

The Control of Vibrations at Work Regulations 2005

Together with all other Acts, Regulations, Orders, ACoP’s and Guidance Notes that may be applicable to the undertakings.

| Project Title: | Waitrose Store, Newport |  |  |  |  |  |  |  |  | Assessed By: |  |  |  |  | Sean Crabtree |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task / Activity: | Insulation of ductwork and heating, domestic and chilled water pipework |  |  |  |  |  |  |  |  | Date Prepared: |  |  |  |  | 17/08/2026 |  |  |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  |  | Risk Rating |  |
| ### Hazard Identified |  |  | E |  | C | P |  | 3 | 2 |  | 1 | 3 | 2 |  |  | 1 | #### B x C = D |  |
| Cuts and injuries from sharp objects |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Risk Assessment Ratings |  | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Gloves shall be selected which are suitable for the task and which fit the hand; - When using saws, knives, etc., hands are to be kept away from the blade at all times and cuts are to be made away from the body; - Adequate time is to be allocated to undertake work with sharp blades or knives. Do not rush the task as accidents can be the result from being hasty or rushing; - Do not use excessive force to either free a sharp edge or to try and cut into a dense substance as this often leads to an uncontrolled slip of the blade resulting in an injury to the user; - Suitable safety gloves are to be worn at all times when moving materials; - All cuts and abrasions are to be treated immediately and suitable first aid given. Cuts and abrasions must be covered by waterproof plasters before returning back to work. All accidents/injuries are to be recorded within the site accident book; - When Stanley knives (or similar) are not in use, blades are to be retracted or stored within a sheath. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (2) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Use of Ladders and Stepladders |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Step ladders will only be used if all other forms of safe access have been considered and have been dismissed as impractical due to the time required to undertake the work or space requirements etc. - Consideration given to the above; step ladders shall be used for short duration works only (approximately 15 minutes) if their use has been considered and authorised by the Site Supervisor; - When used ensure that step ladders are thoroughly inspected prior to use and not used if there are bent or dented rungs or stiles or cracks / splits, inform the Supervisor if faults are found; - Never over-reach, come down and move the steps accordingly; - Step ladders must be suitable for commercial use and shall be of an industrial grade conforming to BS EN 131; - Keep your footwear clean, keep rungs and steps clean and be extra careful if the ladder has become wet; - When working on stepladders, three points of contact must be maintained at all times. Small tools shall be carried on a suitable tool belt; - Persons are not to stand beyond the 3rd step from the top when using step ladders. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Use of a MEWP |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Ensure that the MEWP selected is suitable for the intended task taking into account ground conditions, height to be achieved and SWL; - Ensure that the entrapment device is always located in a position whereby it can be accessed with ease at ground level e.g. if working against a wall ensure the button is not on the side of the machine facing wall; - Only those persons holding an IPAF card for the type of machine being worked on are to operate MEWPs; - Visually inspect the MEWP prior to use. Check that all audible and visual warnings are operational and that there are no faults present; - Ensure that both sets of controls (platform controls and emergency controls) are operational; - Any defects are to be reported and the equipment removed from use until maintenance has been carried out by an authorised person that is trained and competent to do so; - Ensure that the test of thorough examination is on site for the MEWP prior to its use and that the daily inspection log has been completed. - Ensure that the ground conditions are suitable for the MEWP to travel on and that there are no soft areas, covered voids or obstacles that may cause imbalance; - Never lean over the guard rails to view the MEWP base, wheels or tracks while raising the platform; - Ensure that you wear a restraint of the correct length and harness when operating a cherry picker and that the restraint is attached to the correct point on the MEWP - not hand rails etc.; - As required, a traffic marshal is to be used when moving around site to help with manoeuvres; - Ensure sufficient clearance when travelling under or past overhead obstructions taking into account any the platform movements that can occur when travelling; - The key must be removed from any plant / equipment when the operator leaves to stop un-authorised use; - Ensure lighting conditions are adequate in all areas so that the operator is able to notice any overhead obstructions; - Place barriers around the base of the MEWP and under the basket if using a cherry picker to eliminate the hazard of the machine being struck by other vehicles and to prevent any materials from falling from the basket onto third parties. - If outriggers are required then ensure that they are fully extended. If the lack of space prevents them from being extended then do not use the machine, inform your supervisor; - If there are services e.g. overhead power lines etc. in the vicinity of the work are a safe system of work needs to be produced specific for the task; - Never exceed the safe working load by storing too many materials in the basket, know the safe working load of the individual machine and stay well within it e.g. consider the weight of hand tools and personnel; The SWL must be on the side of the machine, if it is not do not use it. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Muscular Skeletal Disorders |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Mechanical means e.g. sack barrow, trolley, pallet forks shall be used if there is a foreseeable risk of injury due to manual handling; - Where hazardous manual handling activities cannot be avoided, the risk of injury shall be reduced as far as is reasonably practicable by: - Trained operatives using correct manual handling techniques; - Reducing loads and separating into smaller loads; - Training shall be provided in the correct methods of lifting; - Operatives may be supervised to ensure that the best available technique of lifting is being used; - Operatives are never to attempt to lift something that is too heavy for their own capabilities; - Ask for assistance or ensure that a number of persons help with a task if the load cannot be broken down to reduce the weight; - Where two-man lifting is adopted then one is to take control and give directions in lifting co-ordination; - Ensure the route is cleared of all trip hazards, obstructions and persons; - Check for sharp edges and cover where possible; - Repetitive bending, twisting and heavy lifting shall be avoided; - Where possible working in poor conditions shall be avoided e.g. too hot, too cold, etc.; - Operatives are encouraged to report symptoms to their supervisor / manager immediately. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Slips and Trips |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - All spillages are to be reported immediately and cleared using the correct equipment to deal with the particular product (refer to the Material Safety Data Sheet and COSHH assessment for substances); - All areas of site must be kept clean and clear of debris; - Operatives shall be appropriately instructed in the importance of reducing slips and trips, maintaining housekeeping and no horseplay; - Operatives are to clear work areas as soon as possible after the waste has been produced. Recycle waste and place in the correct skips; - All waste must be placed into an area which has been defined by the Principal Contractor; - Never overload skips. Report full skips to the Principal Contractor; - Access and egress routes must remain clear at all times and shall not be used as storage areas; - Ensure fire escape routes and exits are kept clear at all times; - There is to be co-operation and co-ordination of activities with all personnel and Site Management to achieve high standards of housekeeping; - Adequate task and general lighting is to be available at all times; - Wear suitable footwear with adequate tread on the sole; - The Site Supervisor shall assess weather conditions which may affect site operations. Particularly during icy, strong winds or heavy rain conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (2) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Substances harmful to health |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Obtain the relevant Material Safety Data Sheets from the supplier/manufacturer and produce a site specific COSHH assessment; - Operatives shall be familiar with the COSHH assessments and shall be made aware of the hazards and controls; - Operatives must not use any substances without having access to the relevant COSHH Assessment; - Where practical low hazard substances shall be used in preference to those that are hazardous; - Suitable PPE shall be used where defined in the COSHH assessment (e.g. masks, gloves, RPE); - Good standards of hygiene are to be maintained. Wash hands before eating, drinking and going to the toilet. Wash hands regularly to remove any substances. Use barrier cream; - Operatives shall be trained in the correct use of PPE and informed of the system for issue and renewal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (2) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Protection of Third Parties |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - If a Principal Contractor has been appointed there shall be full communication and co-ordination of activities with regards to emergency procedures; - The Principal Contractor shall be responsible for assessing and set up of the main site security requirements; - The safety of third parties shall be considered at the commencement of all works. Controls required with regards to the safety of others shall be in place before the works commence e.g. barriers, signage, etc., Restrict entry and clearly define the area in which works are being undertaken; - No third parties shall be allowed in the working area without the express permission from the Supervisor; - All third parties shall wear appropriate PPE while in the work areas and they shall be escorted at all times; - At no time shall materials be stored in access or egress routes or common areas; - Materials unloaded on the road shall be taken directly onto site to prevent injury to third parties. During the unloading there shall be a lookout placed to ensure the protection of third parties during the unloading or loading of vehicles; - The Principal Contractor shall be relied upon to ensure that there is adequate signage and fencing to prevent unauthorised access to the site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |

|  |  | A |  |  |  |  | B |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  | Severity |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  | C | P |  | 3 | 2 | 1 | 3 | 2 |  | 1 | #### B x C = D |  |
| Use of Harnesses |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  | 4 Medium Risk |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Fall arrest equipment in the form of harnesses and shock absorbing lanyards shall be provided and used by trained and competent persons; - A rescue plan must be produced and communicated to the workforce; - If a restraint or inertia line is to be attached to an anchor point ensure that mansafe systems have been tested or the anchor point has been confirmed as suitable by a competent person; - When inertia lines are used ensure that if they are used in the horizontal position that they can be used in this manner as many inertia lines can only be used in a 15o arc directly under the inertia block; - Operatives are only to use harness’s if they are trained and competent in their use having attended a course outlining the types of harness and restraint or arrest systems; - Harnesses must be inspected visually prior to use and a record made of weekly inspections. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |

|  |  | A |  |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Who’s at Risk? |  |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
| ### Hazard Identified |  | E |  |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
| Asbestos Materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - The Client / Principal Contractor must undertake a comprehensive asbestos survey of all areas where works are to be undertaken and the results of the asbestos survey are to be passed to the Principal Contractor and made available to Crabtree Insulation Ltd; - For demolition and refurbishment works, the minimum required survey in accordance with HSE HSG 264 is a Demolition/Refurbishment Survey; - All previous asbestos Management Surveys are to be passed to Crabtree Insulation Ltd for review so that we can assess if there will be any risk during the undertaking of our works; - If we are required to undertake our works in an area where licensed asbestos removal has been completed, we will request confirmation by way of the 4 stage clearance certificate, which should be made available on site; - All persons who could potentially disturb asbestos containing materials during the undertaking of their works will have asbestos awareness training. This is mandatory; - In the event that a material is discovered on site that is believed to be asbestos STOP WORK and tell others working in the area of the suspected danger. Report to the Site Manager the area where the material was found. Do not recommence work until you have been authorised to do so; - Report immediately to your Supervisor any accidental damage to asbestos containing materials during our works; - If you are concerned about potential exposure, report to the office. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |
| I have read and understand the requirements of this Risk Assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  | Signature |  |  |  |  |  |  | Date |  |  |  |  |  |  |